实验六plus、虚拟局域网配置随堂测试

一、实验目的

测试同学们对VLAN配置及交换机命令的掌握情况

二、实验内容

1．画出网络拓扑图；

2．进行VTP的配置；

3．测试VLAN的通信效果。

三、实验步骤

在模拟器中构建一个如下图所示的网络拓扑图，配置三个VLAN，VLAN111用于教师VLAN，VLAN222用于学生VLAN，VLAN333用于其他用户。

请自己写命令完成VLAN的配置，要求将Switch0改名为SW111，Switch1改名为SW222，且SW111为VTP服务器，SW222为VTP客户端，在SW111上配置完VLAN后，SW222将自动获取三个VLAN的配置信息。

下面的命令请同学们自己写，步骤已经给大家写好。

1）配置第一台交换机

创建VLAN

将接口加入到各自的VLAN

配置VLAN Trunk接口

再配置VTP服务器

2）配置第二台交换机

先配置VLAN Trunk接口

再配置VTP客户端

配置好之后发现在VTP服务器SW111上的VLAN配置信息、VTP域名以及VLAN配置内容均已通过中继接口f0/24传播到VTP客户机模式的交换机SW222上了。

下面将接口添加到VLAN

测试：同一VLAN可以ping通，不同VLAN间的不能ping通。

检查要求：

1、IP地址、VLAN名称、交换机名称都要按要求配置；

2、必须有VTP配置；

3、测试结果正确。

Switch0

Switch>en

Switch#configure terminal

Switch(config)#hostname SW111

SW111(config)#vtp domain VLAN_Class

SW111(config)#vtp mode server

SW111(config)#vlan 111

SW111(config-vlan)#name Teacher_VLAN

SW111(config-vlan)#exit

SW111(config)#vlan 222

SW111(config-vlan)#name Student_VLAN

SW111(config-vlan)#exit

SW111(config)#vlan 333

SW111(config-vlan)#name Other_VLAN

SW111(config-vlan)#exit

SW111(config)#interface fastethernet 0/24

SW111(config-if)#switchport mode trunk

SW111(config-if)#exit

SW111(config)#interface fastethernet 0/1

SW111(config-if)#switchport access vlan 111

SW111(config-if)#exit

SW111(config)#interface fastethernet 0/2

SW111(config-if)#switchport access vlan 111

SW111(config-if)#exit

SW111(config)#interface fastethernet 0/3

SW111(config-if)#switchport access vlan 222

SW111(config-if)#exit

SW111(config)#exit

Switch1

Switch>en

Switch#configure terminal

Switch(config)#hostname SW222

SW222(config)#interface fastethernet 0/24

SW222(config-if)#switchport mode trunk

SW222(config-if)#exit

SW222(config)#vtp domain VLAN_Class

SW222(config)#vtp mode client

SW222(config)#interface fastethernet 0/1

SW222(config-if)#switchport access vlan 333

SW222(config-if)#exit

SW222(config)#interface fastethernet 0/2

SW222(config-if)#switchport access vlan 333

SW222(config-if)#exit

SW222(config)#interface fastethernet 0/3

SW222(config-if)#switchport access vlan 333

SW222(config-if)#exit

SW222(config)#interface fastethernet 0/4

SW222(config-if)#switchport access vlan 222

SW222(config-if)#exit

SW222(config)#exit

Ping

Packet Tracer PC Command Line 1.0

C:\>ping 192.168.11.12

Pinging 192.168.11.12 with 32 bytes of data:

Reply from 192.168.11.12: bytes=32 time<1ms TTL=128

Reply from 192.168.11.12: bytes=32 time<1ms TTL=128

Reply from 192.168.11.12: bytes=32 time<1ms TTL=128

Reply from 192.168.11.12: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.11.12:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 0ms, Average = 0ms

C:\>ping 192.168.11.13

Pinging 192.168.11.13 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.168.11.13:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

C:\>ping 192.168.11.15

Pinging 192.168.11.15 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.168.11.15:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

C:\>